Alain BROBECKER                                   Dracula / Positivity. (STe)

rte de Dardagny                                   baah / Arm's Tech. (Archie)

01630 CHALLEX                                                      baah (PC)

FRANCE

This archive normally contains...

KICK_SRC.DOC   Well, guess what.

ASM_ME  .BAT   A simple batch file to show the way all must be assembled.

BAAH    .XXX   The binary datas used in the 4kb demo.

BALL    .XXX

BALL    .PAL

GEN_SPR .INC   The include files.

INITMODX.INC

MEMLIB  .INC

PUT_PAL .INC

MODE0LIB.INC

KICK_ME .S     The main source.

RINCBIN .INC   The utility which allows to use binaries includes.

RINCBIN .COM   (By Philippe Rhode)

Here comes a small explanation about the most interesting things (the less

unintersting ones I mean) that can be found in this small demo, so you

won' t need to go deep in the code if you' re interested.

Oh, by the way, this is my first ever demo on the PC, so no wonder if

loads of things are not fully optimised. Also consider it was made on a

386sx (a monomedia one, arf!) and so, nothing is optimised for cache...

Also, some others things are not optimised and I know how to improve them,

but well, we must stop somewhere! (I especially think to the sprite

clearing routine!) Some people would also say I should have done vram to

vram copy in the sprite routine. True, but I didn' t want a routine which

is usable only in demos.

The first interesting thing is the sprite code generating routine.

Well, you must consider that in a sprite, there are many blank pixels,

and generally you go through the whole sprite (a square), test the pixel

to see if it must be drawn, and eventually draw it if need be. The aim

of generating a sprite routine is to avoid the tests. I go through the

sprite, and for each non null pixel I generate a

'mov ds:[bx+offset_16bits],pixel_8bits'

Where ds:bx points on the position of sprite. (y*80 + x-xmod(4))

If the pixel is null, I only affects the offset for the next pixel.

Well, since we are working in ModeX, I also generate a small part of

code before each plane process which will rotate the map mask register

and affects the original offset given to the generated routine (in bx)

if we have passed from plane 3 to plane 0.

Humm, I have to say that I also generate a word move instead of two

byte moves if possible. (It proves a bit faster, and takes less memory, so..)

I tried to do it with longwords, but it does not work. (If the long is

not accessed on a multiple of 4, it slows things down. At least on my

computer!)

The second thing is the way the Julia fractal is calculated. I got the

trick from Sam Laur's MINIFRAC. (A good miniprog... Strongly recommended)

Generally, you need to calculate for each iteration...

x'=x*x-y*y+juliax

y'=2*x*y+juliay

norm=x*x+y*y

The last formula is used to test the divergence of the serie. Well, quite

simply, make the same test with x*x-y*y, you will have less calculations,

and the result is nicer. (That' s my point of view!)

The last interesting thing is the simulation of collisions between balls!

The result may look ok, but in fact there are some cheats. Normally,

to test if 2 balls are colliding together, you must see if there is

a time when the distance between their centers is exactly r1+r2. (Where

r1 and r2 are the radius of balls.) This is quite easy if you consider

that the movements of both balls are linear. Then, you place the balls

at the position they have at the intersection time, and you change the

velocities acoording to the collision.

The problem may look quite simple, but with you must consider that we

want an animation, and movements are not linear. (gravity...) In fact, you

consider the movements are linear during a small amount of time (for example

one VBl). You calculate the intersection time. Let' s say it is t=0.5

(t=0 -> beginning of vbl t=1 -> end of vbl), so you place the balls

at t=0.5, you change the velocities, and then you can know the position

at t=1. (It' s again a linear movement with modified velocities)

BUT, the problem proves harder if you have more than two balls colliding

during the same VBl. Suppose ball1 will collide with ball2 at t=0.5,

and then ball3 will collide with ball2 at t=0.75. Hey, is it colliding

with the ball2 possessing velocitie before or after collision with ball1?..

Well, quite a big mess! You must test collision for each couple of balls,

then put all balls at the psition they will have at the time of the first

collision, and begin all collisions test anew with the new velocities for

the time which is left...

It' s despairing to find something like this when you are lazy. So I

decided that to test balls collision I won' t use the physical method,

but just see if at t=1 the two balls are intersecting. When a ball

collides another one (or a wall), it will be locked for the whole vbl.

This mean in fact that it won' t move during the vbl of the collision,

I just change the velocity of it.

Well, it proves much easier... I bet there are other methods, so if you

know one, (better than mine, for speed or realism) write it down to me!

Oh, by the way, changements of the velocities is made the following way.

I suppose o1 and o2 are the center of spheres, v1 and v2 their velocities.

When there is a collision I quite simply exchange the compound of v1 and v2

along the o1o2 axis. (So, you will ahve scalar products and you must know

the norm of o1o2 (one sqrt))

Hope this small text has proved usefull to you.. Else, it' s sad but I just

spent half an hour (or something like that) doing something useless.

It seems you have read everything up to here, well it would be nice to send

me a small postcard, so that I would see if this has proved usefull or not!

baah.